| Equipment Owner or Responsible Person |  | Equipment Type/Make/Model/Shape |  | Call Sign/Equipment ID |  | Registration Number |
| --- | --- | --- | --- | --- | --- | --- |
| Company the inspection is conducted for |  | Inspected by (print) |  | Inspection Company |  |  |
| Declaration Submitted | Towing Capacity | Meter Reading | First Inspection |  | Annual Inspection |  |
| Yes No |  |  | Random Inspection |  | Re-Inspection |  |

|  | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Electrical Labelling |  |  |  |
| Main Switch compliant with AS 3000 sign |  |  |  |
| Maximum voltage above ELV within the enclosure Sign |  |  |  |
| Signs on any cover or door which exposes live conductors |  |  |  |
| All fixed switchgear shall be fitted with a sign showing how to carry out resuscitation after an electric shock. |  |  |  |
| Authorised Persons Only Sign |  |  |  |
| Isolation point Sign |  |  |  |
| Protection |  |  |  |
| The minimum degree of protection against access to live parts shall be IP2X |  |  |  |
| All electrical conductors terminated correctly, secured and effective as per AS3000 |  |  |  |
| Lockable isolation of electrical energy when inspection, repair or maintenance is being carried out |  |  |  |
| A maximum of 30 mA RCD/MCB protection |  |  |  |
| RCD Interrupt Active and Neutral |  |  |  |
| Test that the Earth Leakage is functional and trips the relevant breaker |  |  |  |
| Overcurrent Protection |  |  |  |
| Segregation of ELV and LV |  |  |  |
| The LV cables are double insulated |  |  |  |
| All cable or each conductor of a multi-core cable are insulated to the highest voltage |  |  |  |
| The LV cables are physically separated from the ELV cables by suitable continuous barriers |  |  |  |
| General |  |  |  |
| Electrical enclosures and panels must be locked or require a tool to gain access. |  |  |  |
| Bushings on any cable that passes through a metallic structure |  |  |  |
| Positioning of outlets easy to access |  |  |  |
| All Earth cable to be green and yellow |  |  |  |
| Relevant Circuit Diagram |  |  |  |
| Confirm any electrical leads have test-tag in date (elec licence # must be ledgible) |  |  |  |

| Result of Inspection | Pass: | Yes No | Follow up inspection required: | Yes No |  |
| --- | --- | --- | --- | --- | --- |
|  | Sticker Issued (circle) | Green Orange | Sticker number: |  | Expiry date / / |
| Comments or details of repairs needed |  |  |  |  |  |

I certify that this report is a true record of the equipment’s compliance.

Signature (Inspector):		Date: 		Location of Inspection: